Agarose vs Polyacrylamide Gels

|  | Agarose | Polyacrylamide |
| --- | --- | --- |
| Horizontal or vertical? | Horizontal | Vertical |
| One molecule or lots of molecules? | Lots of molecules held together by intermolecular forces | One large molecule |
| Why are they poured horizontal or vertical? | Because you can. It’s easier to pour a horizontal gel than it is to pour a vertical gel. | Need glass plates for structure- very thin gels that need support. Air speeds up polymerization. If the gel was poured open to air, it would not solidify evenly. |
| Separates large molecules or small ones? | Both, but better for large molecules | Both, but better for small molecules |
| Lets you tell small differences between sizes of bands? | No | Yes |
| Can be reheated and poured? | Yes | No |
| Powder is toxic? | No | Yes |
| Solid is toxic | No | No |
| Type of molecules run in this type of gel | Mostly DNA | DNA or proteins |
| Big gaps or small gaps in gel? | Bigger gaps than with polyacrylamide | Smaller gaps |
| Special uses of this gel? |  | Sequencing gels, protein gels |
| Things to know when running the gel | Make sure buffer covers wells when running. If the gel runs too hot, the agarose might melt. | Make sure buffer covers wells when running. Check for leaks before running. |
| Things to know when handling the gel | Gel easily breaks where wells are. | Gel easily rips. |
| Advantages to using this gel | Can re-pour the gel if you goof. Can decide if you want lots of narrow lanes or a few wide lanes. Fewer ingredients to mix. | Easy to stain. Can dry down the gel to save a copy. |
| Disadvantages of using this gel | Can’t tell small differences between band sizes. Bands are usually not as sharp as with polyacrylamide. | Need to know concentration needed before buying the gels. Need to know number of samples so that you can buy enough lanes for samples. If you are making them yourself, they take a long time to make and are challenging to pour. |
| Expense? | Agarose is about $1 a gram- considered expensive. | Gels usually bought pre-poured. Cost $5-7 per gel. |